# Pre-start checks for the MMU

Do these checks at the depot before every shift. Do not leave the depot if any check fails.

## Walk-around

Check the AN tank level on the gauge.

Inspect the auger hoses for cracks and leaks.

Run the emulsion pump for 30 seconds and read the pressure.

Check the gassing solution flow at the test port.

## Limits

| Check | Limit | If outside the limit |
| --- | --- | --- |
| AN tank level | at least 20% | refill at the depot |
| Emulsion pump pressure | 8 to 12 bar | stop and report to the supervisor |
| Auger hoses | no cracks, no leaks | replace before leaving |
| Gassing solution flow | 0.8 to 1.2 L/min | recalibrate the gassing pump |

Sign the MMU logbook when all checks pass.